# PJM RTEP - 2024/25 LONG-TERM MARKET EFFICIENCY WINDOW 1

PROBLEM STATEMENT & REQUIREMENTS

## Document Scope: 2029 RTEP Market Efficiency Congestion

# 2024/25 Long-Term MARKET EFFICIENCY Window 1

### Purpose of Proposal Window

PJM seeks technical solutions, also called proposals, to resolve market efficiency eligible congestion drivers on facilities identified below in accordance with all applicable planning criteria (PJM, NERC, SERC, RFC, and local Transmission Owner criteria) and market efficiency criteria.

#### Criteria Applied by PJM for this Proposal Window

a.   Market Efficiency Criteria:

    - Annual simulated congestion frequency of at least 25 hours in each of the 2029 and 2032 study years.

    - Congestion threshold:

Lower voltage facilities: minimum of $1 million congestion in each of the 2029 and 2032 study years.

Regional facilities: minimum of $10 million congestion in each of the 2029 and 2032 study years.

Interregional facilities: minimum of $0.5 million congestion in each of the 2029 and 2032 study years.

Although the above criteria are met, PJM may not recommend proposals for certain facilities due to following exceptions:

  - Market Efficiency Exceptions:

    - Congestion is significantly influenced by one FSA generator or a set of FSA generators.

    - Majority of the congestion was already addressed in previous window(s).

    - Simulated congestion for future study years displays a declining trend.

    - Any other expected change in the assumptions indicating that congestion may not persist.

### Terminology for Proposal Windows

For market efficiency proposal windows, PJM provides a list of eligible congestion drivers for the relevant study years that were identified through the simulation analysis. The following column headings are generally representative of the data fields that will be used to identify the specific facility among other factors of the output of this analysis. Additional information may be provided upon request by PJM.

Typical market efficiency column headings:

| Column Headings | Title | Description |
| --- | --- | --- |
| FG# | Constraint Number | Identifier used for facility constraint |
| Constraint | Facility name | Description of monitored facility |
| Area | Area | Identifies the PJM transmission zone for the facility. |
| Type | Type | Identifies the type of facility such as a transformer, interface, or a transmission line |
| Binding (Hours) | Frequency | Number of hours the facility is constrained for the annual study year of the simulation |
| Annual Congestion ($Millions) | Congestion | Total annual congestion dollars for the facility for the study year of the simulation |
| Comment | Comment | M2M signifies a market-to-market facility. |

### Analysis Procedure

Participants are expected to develop solutions to the congestion drivers posted for the window and perform analysis to validate that the solutions remove a significant portion of the congestion driver.

The competitive planning process is documented in PJM Manual 14F, which is available here: http://www.pjm.com/-/media/documents/manuals/m14f.ashx.

Proposed solutions must also meet Transmission Owner Planning Criteria, which is available here: http://www.pjm.com/planning/planning-criteria/to-planning-criteria.aspx

The market efficiency process is documented in PJM Manual 14B, which is available here: http://www.pjm.com/-/media/documents/manuals/m14b.ashx.

Although PJM does its best to provide complete and accurate results, changes to the list of congestion drivers under consideration are possible. That is, flowgates may be added or removed from consideration in the proposal window. PJM works with Transmission Owners, Generation Owners, neighboring TOs and other affected parties to verify the quality of the analysis. PJM endeavors to minimize such changes and will clearly communicate any changes to the participants.

PJM regularly updates the system model to reflect changes to the transmission system. Analyses are performed to verify that congestion drivers continue to be valid, no new congestion drivers have appeared and proposed solutions still address the targeted congestion driver(s) and don’t create new violations.

PJM shall determine the more efficient or cost-effective enhancements or expansions for any congestion driver.

### Scope of Work

PJM is seeking proposals to resolve identified market efficiency congestion drivers as demonstrated in the 2024/25 RTEP Base Market Efficiency PROMOD model, the 202425ME PJM Base 2029 v2025-04-10 scenario.

Note: In addition to the congestion drivers posted for this window, congestion has been identified for a number of other facilities where the loading has a higher degree of uncertainty or is expected to be addressed by other future upgrades to the system. PJM is not seeking competitive proposals to address these congested facilities at this time:

- Constraints where the 2025 Load Sensitivity shows significant congestion changes (increases/decreases) due to increased load in parts of the system.

- Constraints with congestion expected to be addressed by future network upgrades, supplemental projects or upgrades on the neighboring systems.

- Coordinated PJM/MISO Market-to-Market constraints on facilities located in MISO.

- Constraints that may see significant congestion changes from MISO Tranche 1 and Tranche 2.1 projects.

### Market Efficiency Objectives

- Develop complete solutions to address the identified eligible congestion drivers.

- Perform and compare market simulations with and without proposed enhancements or expansions to evaluate if the benefit/cost ratio is at least 1.25 using the criteria as defined in Schedule 6, §1.5.7 of the PJM Operating Agreement and PJM Manual 14B, Attachment E.

- Perform high level reliability analysis of proposed market efficiency enhancements or expansions to ensure the proposed enhancement or expansion does not create any reliability issues.

- Develop solutions to address all new criteria violations or congestion drivers generated as a consequence of proposed solution. Solutions to these secondary violations or congestion drivers are required for the proposal in order to be considered.

- Adhere to all applicable planning criteria, including PJM, NERC, SERC, RFC and Local Transmission Owner Criteria.

- Only proposals which address one or more of these PJM identified eligible congestion drivers will be evaluated. If the proposal does not significantly address a PJM eligible congestion driver, and is otherwise substantially deficient or is seriously flawed, it will be rejected and the proposer will be notified.

#### WHAT PJM PROVIDES:

The information listed below is provided to allow replication of PJM analyses. Some of this data is designated Critical Energy Infrastructure Information (CEII) and must be handled consistent with PJM’s CEII request process at Competitive Planning Process page on the PJM website:

### Modeling Data for the Market Efficiency Criteria:

The following data and related information is provided for this proposal window through the PJM Competitive Planning Process or RTEP Development websites or in the PJM Transmission Expansion Advisory Committee (TEAC) materials.

1.   Base Case Market Efficiency Economic Models: These models contain the base set of PROMOD data for the 2024/2025 Market Efficiency Analysis. Access to these models requires CEII authorization along with an active license with HITACHI ENERGY for PROMOD and Nodal Simulation Data.

PROMOD case and supporting files are available under the Market Efficiency Economic Planning Models section associated with the 2024/25 Long-Term Market Efficiency Window 1: https://www.pjm.com/planning/rtep-development/market-efficiency/economic-planning-process.aspx.

- Market Efficiency Eligible Congestion Drivers: Proposals must provide significant congestion reduction for facilities identified as eligible congestion drivers.

The list of eligible congestion drivers can be obtained here: https://pjm.com/planning/competitive-planning-process.aspx.

- 2024/25 Market Efficiency Input Assumption Details: This file contains the details of the input assumptions used for the 2024/2025 Market Efficiency Analysis.

https://www.pjm.com/-/media/DotCom/planning/rtep-dev/market-efficiency/2024-2025-pjm-market-efficiency-base-case-input-assumption-details.xlsx

### Other Supporting Market Efficiency files:  The following supporting files can be downloaded from the secure Market Efficiency page:

https://www.pjm.com/planning/rtep-development/market-efficiency/economic-planning-process.aspx

### 2024/25 Market Efficiency Base Congestion Results: For informational purposes only, PJM provided the complete set of base case congestion results for simulated years 2029 and 2032.

  - 2024/25 ARR Model: This file provides the ARR modeling data for the 2024/2025 Market Efficiency Analysis.

  - 2024/25 ME Window 1 Benefit/Cost Evaluation Tool: This file contains the inputs and formulas necessary for calculating the B/C Ratios.

  - 2029 Scenario Sample PROMOD Hourly Output Files: These zip files contain sample inputs and output for a test data set.

  - 2029 SP RAW File:  This file contains the RAW Power Flow input file used in the 2029 Market Efficiency Base Case.  PJM does not post separate PSSE reliability power flow files for the market efficiency windows. This power flow should be strictly used for topology evaluation, and it may not be suitable for reliability analyses. Reliability power flows, closely matching what was imported in PROMOD, can be provided by PJM Transmission Planning upon request.

  - Market Efficiency Sensitivity Files:  These files contain data allowing for PROMOD sensitivities to be created from the PROMOD Base Case Economic Models.

  - Procedure for Executing 2024/25 Base Case PROMOD Simulations: This document describes the PROMOD simulation setup steps to run the PJM Market Efficiency base case models.

  - PROMHMC.CFG: A configuration file described in the Procedure for Executing 2024/25 Base Case PROMOD Simulations file.

### Response Back To PJM (Deliverables)

This section describes the required elements of a complete proposal. The absence of any element renders the proposal incomplete and the proposal will not be considered for selection.

Often there are several viable solutions to a given violation. Include alternate approaches in separate proposals. PJM will not accept proposals with multiple options.

Four categories of information are described below: three are required for a complete package and one is conditional.

- Technical analysis files and documentation

- Completed proposal submittal template

- Project diagrams

- Company evaluation and operations and maintenance information (if required)

#### Technical Analysis Files and Documentation

Include the following technical information to provide a complete project proposal package:

- A detailed analysis spreadsheet showing the planning analysis results for the project.

- A set of updates to the power flow cases which model the proposed solution. Provide files in a format compatible with PSS/E version 35.3. Provide only solvable and convergent solutions. Include an idv, or equivalent type, file in order to apply changes to other models. Assign a unique identifier when new busses are required. Provide contingencies in a single file for each contingency type. Organize the contingencies into one of three categories:

  - Modified Contingencies

  - New Contingencies

  - Deleted Contingencies

- Short circuit .CHF file or .DXT file compatible with ASPEN 15

- List of all proposed equipment along with relevant parameters and assumptions.

  - All necessary parameters, e.g., equipment ratings, impedances, line lengths, etc.

  - For reactive devices, settings and outputs

  - For synchronous machines, MW and MVAR output assumptions

- An analysis report of proposed solution which identifies the issues being addressed, as well as any additional benefits that the proposed solution can provide, including but not limited to addressing long lead issues.

- Additional documentation required to verify the proposal.

### PJM Proposal Submittal

The PJM Competitive Planner Tool is used for proposal(s) submittal and captures project details, such as the criteria violations or system constraints that are being targeted by the proposed solution, the overall and specific project descriptions and the details of cost commitment, if included. The Competitive Planner Tool and user guide are available on the Competitive Planning Process page: https://www.pjm.com/planning/competitive-planning-process.aspx.

All proposals will be made public and posted on pjm.com after the proposal window is closed. Mark all CEII and business confidential information for redaction from this public posting. Redact only information which meets the criteria of CEII or business proprietary and confidential information.

Redaction guidelines can be found in PJM Manual 14F, Section 6.2. Please note that this section provides guidance only. PJM reserves the right to challenge proposed redaction of information in order to ensure the appropriate level of transparency.

#### Project Diagrams

Provide project diagrams in the PJM Competitive Planner Tool to detail how the proposed solution will modify existing infrastructure, how new infrastructure will be configured and where new infrastructure will be sited. Project diagrams should include, but are not limited to, the following detail:

- Single line diagrams

- Substation general arrangement and station layout. If expanding the substation, identify the following:

  - Area to be modified

  - Land ownership or acquisition plan

- Line routing diagram:

  - Identify proposed route of new or upgraded transmission lines

  - Clearly identify where  acquisition of additional right-of-way is required

- Detailed project schedule. Include, at minimum, the following major work activities:

  - Engineering and Design

  - Siting and Permitting

  - ROW and Land acquisition

  - Material procurement

  - Construction

  - Testing/Commissioning

Project Capital Expenditure, Cost Containment Information and Cost Escalation

In accordance with Manual 14-F Section 8.4, PJM will initiate the comparative cost framework to evaluate the costs of project proposals that are submitted through PJM’s competitive proposal window process, with the final comparative cost framework being performed once project proposals are found to pass an engineering screen. The comparative cost framework is a multi-step process that calculates project costs and permits the comparison of costs among projects addressing the same violation(s) or constraint(s) (competing projects) submitted through the proposal window.

A cost commitment provision submitted as part of a project proposal may include, but is not limited to, the capital structure (debt to equity ratio) and caps on: initial capital costs (total costs associated with bringing the project into service); the annual revenue requirement; the rate of return on equity (ROE); the debt cost; the total capital cost; allowance for funds used during construction (AFUDC); construction work in progress (CWIP); abandonment costs and schedule guarantees. A cost commitment proposal may also exclude defined cost elements from the cost commitment provision.

Within the Competitive Planner application, there is a Project Financial Information section that includes a financial form through which projected capital expenditures for the project should be provided. This form must be completed and submitted with each proposal submittal.

In addition, following the close of this Window, PJM may request additional details via email regarding the capital structure of submitted projects, and the cost containment provisions proposed in order to perform the comparative cost assessment of the competing proposals.

For the purpose of the escalation calculations in this Proposal Window, PJM recommends the use of a 2.1% escalation factor to ensure consistency in escalation calculations for all submitted proposals.

#### Company Evaluation and Operations and Maintenance Information

For proposers seeking Designated Entity status, provide additional information which will aid PJM in understanding how the proposed solution will be developed, constructed, operated and maintained. Include this information in the PJM Competitive Planner Tool.

Relevant PJM Operating Agreement, Schedule 6, section 1.5.8(f).

Entity-Specific Criteria Considered in Determining the Designated Entity for a Project.  In determining whether the entity proposing a Short-term Project, Long-lead Project or Economic-based Enhancement or Expansion recommended for inclusion in the plan shall be the Designated Entity, the Office of the Interconnection shall consider:  (i) whether in its proposal, the entity indicated its intent to be the Designated Entity; (ii) whether the entity is pre-qualified to be a Designated Entity pursuant to Operating Agreement, Schedule 6, section 1.5.8(a); (iii) information provided either in the proposing entity’s submission  pursuant to the Operating Agreement, Schedule 6, section 1.5.8(a) or 1.5.8(c)(2) relative to the specific proposed project that demonstrates:  (1) the technical and engineering experience of the entity or its affiliate, partner, or parent company, including its previous record regarding construction, maintenance, and operation of transmission facilities relative to the project proposed; (2) ability of the entity or its affiliate, partner, or parent company to construct, maintain, and operate transmission facilities, as proposed, (3) capability of the entity to adhere to standardized construction, maintenance, and operating practices, including the capability for emergency response and restoration of damaged equipment; (4) experience of the entity in acquiring rights of way; (5) evidence of the ability of the entity, its affiliate, partner, or parent company to secure a financial commitment from an approved financial institution(s) agreeing to finance the construction, operation, and maintenance of the project, if it is accepted into the recommended plan; and (iv) any other factors that may be relevant to the proposed project, including but not limited to whether the proposal includes the entity’s previously designated project(s) included in the plan.

#### Transmittal of Proposals

Proposal submissions along with all files required for submission will be done with the PJM Competitive Planner Tool for each submitted proposal.

### Proposal Fees

All proposals, upgrade and greenfield solutions, with the exception of solutions submitted with an estimated cost of $5M or less, submitted for consideration in the 2024/2025 Long-Term Market Efficiency Window 1 are subject to an initial $5,000 deposit per the PJM Operating Agreement 1.5.8(c).

Relevant PJM Operating Agreement language in Section 1.5.8(c):

(c)(1)	All proposals submitted in the proposal windows must contain:  (i) the name and address of the proposing entity; (ii) a statement whether the entity intends to be the Designated Entity for the proposed project; (iii) the location of proposed project, including source and sink, if applicable; (iv) relevant engineering studies, and other relevant information as described in the PJM Manuals pertaining to the proposed project; (v) a proposed initial construction schedule including projected dates on which needed permits are required to be obtained in order to meet the required in-service date; (vi) cost estimates and analyses that provide sufficient detail for the Office of Interconnection to review and analyze the proposed cost of the project; and (vii) with the exception of project proposals submitted with cost estimates of $5 million or less, a $5,000 non-refundable deposit must be included with each project proposal submitted by a proposing entity that indicates an intention to be the Designated Entity.

(c)(1)(i)  In addition, any proposing entity indicating its intention to be the Designated Entity will be responsible for and must pay all actual costs incurred by the Transmission Provider to evaluate the submitted project proposal.  To the extent the Transmission Provider incurs costs to evaluate multiple submitted project proposals where such costs are not severable by individual project proposal, the Transmission Provider shall invoice equal shares of the non-severable costs among the project proposals that cause such non-severable costs to be incurred.  Notwithstanding this method of invoicing non-severable costs, non-severable costs will be jointly and severally owed by the proposing entities that cause such costs to be incurred.

(c)(1)(ii)  All non-refundable deposits will be credited towards the actual costs incurred by the Transmission Provider as a result of the evaluation of a submitted project proposal.

(c)(1)(iii)  Following the close of a proposal window but before the Transmission Provider incurs any third-party consultant work costs to evaluate a submitted project proposal, the Transmission Provider will issue to the proposing entity an initial invoice seeking payment of estimated costs to evaluate each submitted project proposal.  The estimated costs will be determined by considering the:  potential cost of consultant work, historical estimates for project proposals of similar scope, complexity and nature of the need, and/or technology and nature of the project proposal. The Transmission Provider may issue additional invoices to the proposing entity prior to the completion of the evaluation activities associated with a project proposal if the Transmission Provider receives updated actual cost information and/or upon consideration of the factors specified in this section.

(c)(1)(iv)  At the completion of the evaluation activities associated with a project proposal, the Transmission Provider will reconcile the actual costs with monies paid and, to the extent necessary, issue either a final invoice or refund.

(c)(1)(v)  The proposing party must pay any invoiced costs within fifteen (15) calendar days of the Transmission Provider sending the invoice to the proposing entity or its agent.  For good cause shown, this fifteen (15) calendar day time period may be extended by the Transmission Provider.  If the proposing entity fails to pay any invoice within the time period specified and/or extended by the Transmission Provider in accordance with this section, the proposing entity’s pre-qualification status may be suspended and the proposing entity will be ineligible to be a Designated Entity for any projects that do not yet have an executed Designated Entity Agreement.  Such a suspension and/or ineligibility will remain in place until the proposing entity pays in full all outstanding monies owed to the Transmission Provider as a result of the evaluation of the proposing entity’s project proposal(s).

The non-refundable deposit is due at the time of submission. Pay the non-refundable deposit by wire transfer to PJM Interconnection at:

| Wire transfer funds to: | PJM Interconnection, LLC |
| --- | --- |
| Bank Name: | PNC Bank, N.A. New Jersey |
| ABA Number: | 031207607 (Fed Wire and ACH) |
| Account Number: | 80-1358-9826 |
| Provide Reference*: | Order 1000 |

To ensure the non-refundable deposit is properly accounted for, include a reference to “Order 1000 – Competitive submittal PJM Id ###” in the subject, notes or addenda field of the wire transfer form.  Please note that the PJM ID ### will be displayed on the confirmation email you receive after submitting the proposal and is also identified on the competitive planner tool once a competitive proposal is initially saved.

The proposer also needs to provide the following information to PJM:

Company Info, Primary contact name, Billing email & Tax ID#.

The PJM Proposal ID can be located on the Existing Proposal screen in the PJM Competitive Planner Tool

### Timeline

4/11/2025: Opening of 2024/25 Long-Term Market Efficiency Window 1

6/10/2025: Close of 2024/25 Long-Term Market Efficiency Window 1 at 5:00 p.m. Eastern Daylight Time

Notes:

- Confidentiality of individual proposals will be maintained for the duration of the window.

- Proposals received after close of the proposal window will not be accepted.

### Questions

Submit all questions about the proposal window to the PJM Planning Community. Submit questions involving confidential information or CEII under the “Confidential” topic on the Planning Community. Answers will be provided to all participants in the proposal window.

Please reference 2024/25 RTEP Long-Term Proposal Window 1 in all correspondence.

### Specialized Requirements

PJM seeks proposals to evaluate the identified needs in this window.  PJM has not determined specialized requirements that PJM is seeking in project proposals in this window for consideration in its evaluation.

Notice

In accordance with the Open Access Transmission Tariff, Attachment M, the MMU has access to all data submitted to PJM through PJM’s competitive proposal window process.  See Manual 14F, section 8.4.4.

### Document Revision History

4/11/2025 – V1 – 	Original Problem Statement posted to the PJM Competitive Planning Process webpage: https://www.pjm.com/planning/competitive-planning-process.aspx.

.